Customer Complaint:

The vehicle’s horns stay on continuously. The owner thinks the horn relay may be stuck in the on position.  Specifically describe how you would test the horn relay in this case.  Additionally, besides the horn relay, describe three more circuit faults that would cause the horns to remain on. Indicate specifically how you would test for each fault and the action needed to correct each problem.

1999 Plymouth Prowler Horn Circuit

Possible Circuit Problems

Horn switch shorted to ground

Clockspring shorted to ground

Body Control Module

Shorted to ground

Commanding horns on due to inputs

Relay BK/RD control wire grounded between pin 24 & Clockspring pin 5

Horn DG/RD power wire shorted to power